# Final Year Project Report

## Smart AI‑Driven Fire Detection and Suppression System

Authors:

Maya Fakih

Jana Mneimneh

Manuel El Danaf

Johnny El Danaf

Supervisor:

Dr. Gilbert Tekli

Issam Fares Faculty of Technology

Souk El Gharb, Lebanon
January 8, 2026

## Abstract

Fire hazards represent a major threat to human life, infrastructure, and the environment. Conventional fire detection systems based on smoke, heat, or gas sensors often suffer from delayed activation, high false‑alarm rates, and an inability to assess fire severity or progression. In recent years, computer vision and artificial intelligence have emerged as powerful alternatives, enabling earlier detection and richer situational awareness. However, most vision‑based fire detection systems focus primarily on identifying the presence of flames, without evaluating the danger level, growth dynamics, or suppression requirements of a fire event.

This project presents a Smart AI‑Driven Fire Detection and Suppression System that integrates deep learning–based vision models with classical image processing and decision‑support reasoning. The proposed system builds upon the YOLOv8 object detection architecture for real‑time fire localization and augments it with a novel fire‑state assessment layer inspired by dynamic system behavior analysis. Instead of binary fire detection, the system evaluates temporal and spatial fire characteristics to estimate severity and guide response actions. Classical computer vision techniques such as edge detection, Histogram of Oriented Gradients (HOG), and Aggregated Channel Features (ACF) are reviewed and positioned as complementary mechanisms that enhance robustness and early warning capability.

Furthermore, the system design is informed by prior work on multispectral sensing and autonomous fire suppression, demonstrating how intelligent vision can be coupled with actuation and control. The report details the system architecture, literature foundations, methodology, hardware and software design, budget analysis, and future expansion paths. The proposed framework aims to bridge the gap between accurate fire detection and intelligent, actionable fire response.

Contents

Final Year Project Report	1

Smart AI‑Driven Fire Detection and Suppression System	1

Abstract	2

Chapter 1	3

Introduction	3

1.1 Background	3

1.2 Problem Statement	3

1.3 Objectives	3

1.4 Project Contributions	4

Chapter 2	4

Literature Review	4

2.1 Conventional Fire Detection Systems	4

2.2 Classical Computer Vision Techniques for Fire Detection	4

2.3 Machine Learning and Deep Learning Approaches	5

2.4 Multispectral and Autonomous Fire Suppression Systems	5

2.5 Literature Summary and Research Gap	5

Chapter 3	6

System Overview and Architecture	6

3.1 Overall System Architecture	6

3.2 Functional Block Description	6

3.3 System Workflow	6

Chapter 4	7

Methodology	7

4.1 Mechanical Design	7

4.2 Electrical and Embedded Design	7

4.3 Software Design and Implementation	7

Chapter 5	7

Budgeting and Cost Analysis	7

5.1 Core Processing Unit	8

5.2 Camera and Thermal Sensing Options	8

5.3 Budget Configurations	9

Chapter 6	9

Discussion and Evaluation Strategy	9

6.1 Spatial Awareness and Fire Localization Gap	9

6.2 Proposed World-Frame Mapping and IoT Integration	10

6.3 Robotic Nozzle Control and Coordinate Transformation	10

6.4 Separation of AI Vision and Deployment Platforms	10

6.5 System Evolution and Research Continuity	10

Chapter 7	11

Conclusions and Future Work	11

References	12

List of Tables

Table 1 Core Processing Unit	11

Table 2 Camera and Thermal Sensing Options	12

Table 3 Budget Configurations	12

Figure 1_system_architecture	9

## Introduction

### 1.1 Background

Fire detection and suppression systems play a critical role in ensuring safety across residential, industrial, and public environments. Traditional systems rely on point‑based sensors such as smoke detectors, heat sensors, or gas detectors. Although these solutions are widely deployed and relatively inexpensive, they are limited by delayed response times, susceptibility to environmental noise, and the inability to provide spatial information about the fire source. In many cases, alarms are triggered only after a fire has grown significantly, reducing the effectiveness of early intervention.

Advances in artificial intelligence and computer vision have enabled the use of cameras as intelligent sensors capable of detecting fire at early stages. Vision‑based systems can analyze flame color, shape, motion, and intensity, providing richer information than conventional sensors. Nevertheless, many existing AI‑based fire detection approaches treat fire as a binary event, focusing solely on detection accuracy rather than understanding fire behavior, progression, or risk.

### 1.2 Problem Statement

Despite the success of deep learning in visual fire detection, current systems exhibit three major limitations. First, they lack contextual awareness regarding fire severity and growth. Second, they often operate independently of suppression or control mechanisms. Third, they are vulnerable to false detections in complex environments without complementary validation mechanisms. These limitations restrict the deployment of AI fire detection systems in safety‑critical applications.

### 1.3 Objectives

The objectives of this project are: - To design a real‑time fire detection system using AI‑based computer vision. - To extend detection toward fire state and severity assessment. - To integrate classical image processing as complementary validation layers. - To propose an architecture compatible with automated or assisted fire suppression. - To develop a scalable and cost‑effective embedded solution.

### 1.4 Project Contributions

The main contributions of this project include: - A hybrid fire detection framework combining YOLOv8 with decision‑support reasoning. - A fire‑state evaluation mechanism based on temporal and spatial analysis. - A structured integration of classical computer vision and modern deep learning. - A system‑level design suitable for intelligent fire suppression platforms.

## Chapter 2

## Literature Review

### 2.1 Conventional Fire Detection Systems

Conventional fire detection systems primarily rely on smoke, heat, and gas sensors. While reliable in controlled indoor environments, these sensors suffer from delayed activation and frequent false alarms caused by dust, steam, or ambient temperature variations. Moreover, they provide no information about fire location, size, or growth, limiting their usefulness in intelligent response systems.

### 2.2 Classical Computer Vision Techniques for Fire Detection

Early vision‑based fire detection methods utilized handcrafted features and image processing techniques. Edge detection is among the earliest approaches, identifying sharp intensity variations that correspond to flame boundaries. Due to the irregular and dynamic nature of flames, edge‑based methods can capture fire contours; however, they are highly sensitive to illumination changes and reflections, leading to false detections in complex scenes (Kakde et al., 2018; Nainar et al., 2022).

Histogram of Oriented Gradients (HOG) improves upon simple edge detection by analyzing gradient orientation distributions within localized image regions. By capturing both magnitude and direction information, HOG offers improved robustness to lighting variations. Nevertheless, its performance degrades when flame shapes vary significantly or when scenes are cluttered (Dalal & Triggs, 2005).

Aggregated Channel Features (ACF) further enhance detection by combining multiple channels, including color, gradient magnitude, and orientation. This multi‑feature representation enables faster and more accurate detection compared to single‑feature methods, particularly on embedded platforms. Despite these advantages, ACF‑based systems still require careful training and are best used as complementary layers alongside learning‑based models (Dollár et al., 2012; Dollár et al., 2014).

Early fire detection systems also relied on color and shape analysis using classical image processing techniques. Kakde et al. (2018) proposed a real-time fire detection method based on HSV color space segmentation combined with contour extraction and morphological filtering to localize flame regions. While effective under controlled conditions, such approaches remain sensitive to illumination changes and visually similar objects, limiting their robustness in complex environments.

### 2.3 Machine Learning and Deep Learning Approaches

Machine learning approaches introduced classifiers trained on handcrafted features to improve fire detection reliability. More recently, deep learning models, particularly convolutional neural networks (CNNs), have demonstrated superior performance by learning hierarchical fire representations directly from data. Object detection frameworks such as YOLO enable real‑time flame localization with high accuracy. However, most deep learning‑based fire detection systems focus on detection confidence and overlook fire evolution and risk assessment.

Machine learning has also been applied to fire detection using IoT-based sensor networks rather than visual data. Jiji et al. (2022) presented a forest fire detection system that employs wireless sensor nodes measuring environmental parameters and machine learning classifiers such as ANN and SVM for early fire warning. Although effective for large-scale monitoring, this approach does not provide visual localization or fire progression analysis, highlighting the complementary role of vision-based deep learning systems.

Recent studies have further extended intelligent fire detection toward autonomous response systems. Deng et al. (2026) proposed a vision-sharing-based autonomous firefighting robot navigation framework that relies on external CCTV networks rather than onboard perception sensors. The study highlights the limitations of SLAM, LiDAR, and onboard vision in fire environments due to smoke, heat, and sensor degradation, and demonstrates how shared visual intelligence can guide autonomous navigation more reliably. This work supports the separation of perception and deployment platforms in intelligent fire response architectures.

### 2.4 Multispectral and Autonomous Fire Suppression Systems

Advanced research has explored autonomous fire suppression using multispectral sensing. Systems integrating ultraviolet flame detectors and infrared thermal cameras achieve robust detection in low‑visibility environments such as smoke‑filled spaces. Infrared stereovision enables three‑dimensional fire localization, while feedback control and visual servoing allow adaptive nozzle targeting during suppression. Experimental results demonstrate faster and more reliable fire extinguishment compared to manual intervention, particularly in hazardous environments (McNeil & Lattimer, 2016; McNeil et al., 2013).

These studies emphasize the importance of coupling detection with control and feedback mechanisms rather than treating fire detection as an isolated task.

### 2.5 Literature Summary and Research Gap

The reviewed literature demonstrates significant advances in fire detection accuracy and the development of automated suppression systems. However, a gap remains in integrating reliable fire detection with structured decision-making and response planning. While recent studies have explored autonomous or vision-guided firefighting platforms, most existing systems either focus on detection confidence or rely on environment-specific control strategies, with limited emphasis on fire severity assessment and dynamic risk evaluation. This project addresses this gap by integrating AI-based fire detection with fire-state and severity assessment logic to support intelligent, adaptive response.

## Chapter 3

## System Overview and Architecture

### 3.1 Overall System Architecture

The proposed system consists of three main layers: a vision sensing layer, an AI and decision‑support layer, and a response layer. A camera continuously captures video frames, which are processed by the AI module to detect fire regions. Detected fire data is then analyzed temporally to estimate severity and guide alerts or suppression actions.

### 3.2 Functional Block Description

The vision module handles image acquisition and preprocessing. The AI module performs fire detection using YOLOv8 and computes fire metrics. The decision‑support module evaluates fire state and selects appropriate responses. The response layer triggers alarms or interfaces with suppression hardware.

### 3.3 System Workflow

- Continuous image acquisition.

- Preprocessing and normalization.

- Fire detection and localization.

- Fire state evaluation (real vs fire images by detecting heat or smoke)

- Alert generation or suppression activation.

Figure 1_system_architecture

Chapter 4

## Methodology

### 4.1 Mechanical Design

The mechanical design focuses on stable camera mounting, protective housing, and optional actuator integration. Proper field‑of‑view coverage and environmental protection are prioritized to ensure reliable operation. In addition, it explores the ability to benefit from already mounted security cameras for easier industrial integration.

### 4.2 Electrical and Embedded Design

The electrical subsystem includes an embedded AI processor, camera interface, power regulation, and optional smoke or infrared sensors for real fire detection. Emphasis is placed on reliability, modularity, and low power consumption.

### 4.3 Software Design and Implementation

#### 4.3.1 Data Acquisition and Preprocessing

Video frames are captured in real time using a raspberry pi ai camera (might use existing wall mounted security camera for industrial expansion and budget efficiency) resized for efficient inference. Noise reduction and color normalization improve robustness under varying lighting conditions.

#### 4.3.2 AI Vision Module (YOLOv8)

YOLOv8 is employed for real‑time fire detection and localization. The model outputs bounding boxes and confidence scores for detected fire regions.

#### 4.3.3 Fire State Assessment and Decision Support

Fire severity is estimated by analyzing bounding box growth, intensity variation, and temporal persistence. These parameters are combined into a fire state index that enables graduated response rather than binary activation.

#### 4.3.4 Suppression and Alert Logic

Based on the estimated fire state, the system triggers alarms, notifications, or suppression commands. The logic supports escalation and adaptive response.

## Chapter 5

## Budgeting and Cost Analysis

The proposed system is intentionally designed to be low-cost and modular, relying on commercially available components commonly used in smart surveillance and embedded AI applications. The budget focuses primarily on a single embedded processing unit and a thermal or infrared-capable camera, allowing flexibility depending on performance requirements and deployment constraints.

### 5.1 Core Processing Unit

| Component | Description | Estimated Cost (USD) |
| --- | --- | --- |
| Raspberry Pi 5 | Main embedded processing unit for AI inference, IoT communication, and system control | 150 |

Table 1 Core Processing Unit

The Raspberry Pi 5 is selected due to its improved CPU performance, support for hardware acceleration, and compatibility with a wide range of camera and sensor modules. It provides sufficient computational capability to run YOLOv8-based inference while maintaining low power consumption.

### 5.2 Camera and Thermal Sensing Options

The system supports multiple sensing configurations depending on the required level of thermal resolution, cost constraints, and deployment environment. The following options are compatible with the proposed architecture:

| Sensor / Camera Option | Resolution | Interface | Typical Cost (USD) | Notes |
| --- | --- | --- | --- | --- |
| Standard Security Camera with IR | HD / Full HD | USB / CSI | 30–60 | Commonly used for surveillance; supports night vision using IR illumination |
| AMG8833 Thermal Imager | 8×8 (64 pixels) | I2C | ~60 | Low-cost thermal sensing; suitable for coarse heat detection |
| MLX90640 Thermal Camera | 32×24 (768 pixels) | I2C | 60–90 | Higher resolution thermal imaging; available in standard and wide-angle variants |
| Waveshare Long-Wave IR Thermal Module | 80×62 pixels | USB / HAT | ~80 | Higher refresh rates (up to 25 FPS); good balance between cost and performance |
| FLIR Lepton Thermal Module | Higher resolution (model-dependent) | SPI / USB | 200+ | Professional-grade thermal imaging; higher cost but superior accuracy |
| Raspberry PI ai camera (best option) | 12.3 Mega pixels | CSI | ~70-90 | Integrated 12.3MP Sony IMX500 sensor with onboard ML processing |

Table 2 Camera and Thermal Sensing Options

### 5.3 Budget Configurations

Depending on the chosen sensing configuration, the total system cost varies:

- Minimum configuration (Raspberry Pi 5 + standard IR security camera): ~180–210 USD

- Mid-range thermal configuration (Raspberry Pi 5 + MLX90640 or Waveshare module): ~220–250 USD

- Advanced configuration (Raspberry Pi 5 + FLIR Lepton): 350 USD and above

This modular budgeting approach allows the proposed AI fire detection and assessment system to be deployed across a wide range of applications, from low-cost smart buildings to more advanced robotic or industrial firefighting platforms.

| Component | Qty | Cost (USD) |
| --- | --- | --- |
| Embedded AI Board | 1 | 120 |
| Camera Module | 1 | 30 |
| Power Management | 1 | 25 |
| Mechanical Housing | 1 | 40 |
| Actuators / Valves | 1 | 60 |
| Miscellaneous | – | 25 |
| Total |  | 300 USD |

Table 3 Budget Configurations

## Chapter 6

## Discussion and Evaluation Strategy

### 6.1 Spatial Awareness and Fire Localization Gap

While the proposed system successfully detects fire and evaluates its severity, an important research gap is the explicit estimation of fire location within a global spatial context. Current implementation focuses on visual detection and temporal fire-state assessment, without directly mapping detected fire regions to real-world coordinates. Addressing this limitation is essential for enabling precise, automated suppression using robotic or distributed actuation systems.

### 6.2 Proposed World-Frame Mapping and IoT Integration

To overcome this limitation, future system extensions will incorporate a spatial mapping framework based on a fixed world coordinate frame. Each security camera within the monitored environment will be calibrated and registered relative to this global frame. Camera pose estimation and homography or 3D reconstruction techniques will be used to convert image-space fire detections into real-world coordinates.

Detected fire locations will be expressed as (x, y, z) coordinates in the world frame. These coordinates can then be transmitted through an IoT-based communication layer to downstream actuators, including robotic fire-suppression arms, mobile robots, or aerial platforms.

### 6.3 Robotic Nozzle Control and Coordinate Transformation

Each fire-suppression unit, such as a two-degree-of-freedom robotic nozzle or arm, will maintain its own local coordinate frame. Using known transformations between the world frame and each robot’s local frame, the system will convert global fire coordinates into local target positions. Inverse kinematics (IK) will then be applied to compute the required joint angles for accurate nozzle orientation toward the fire source.

This modular approach allows multiple suppression units to respond independently to the same fire event, improving scalability and redundancy.

### 6.4 Separation of AI Vision and Deployment Platforms

A key design principle of this research is the separation between the AI fire detection model and its deployment platform. The proposed YOLOv8-based fire detection and fire-state assessment model is designed to operate as a standalone module. It can be deployed independently to:

- Trigger conventional sprinkler or alarm systems

- Serve as the perception module for robotic fire suppression systems

- Be integrated into fire-detection drones or mobile robots

- Operate within a distributed IoT-based smart building infrastructure

This decoupling ensures that the AI model remains reusable, scalable, and adaptable across different applications without modification to its core architecture.

### 6.5 System Evolution and Research Continuity

The current work represents the first stage of a broader research effort. The initial focus is on robust fire detection and severity assessment. Subsequent versions will incorporate spatial localization, robotic control, and closed-loop suppression feedback. This staged development strategy enables progressive validation while maintaining system reliability and academic clarity.

## Chapter 7

## Conclusions and Future Work

This project demonstrates an AI‑driven fire detection and suppression framework that extends beyond binary detection. By integrating deep learning with decision‑support reasoning, the system provides enhanced situational awareness and a foundation for autonomous fire response. Future work includes multispectral sensor integration, large‑scale deployment, and advanced control strategies.

## References

Dalal, N., & Triggs, B. (2005). Histograms of oriented gradients for human detection. Proceedings of the IEEE Computer Society Conference on Computer Vision and Pattern Recognition (CVPR), 886–893. https://doi.org/10.1109/CVPR.2005.177

Deng, R., Ding, S., Ding, Y., Wang, M., Huang, X., & Usmani, A. S. (2026). Autonomous navigation of non-perception firefighting robot through CCTV-informed vision sharing. Expert Systems with Applications, 296, 129210.https://doi.org/10.1016/j.eswa.2025.129210

Dollár, P., Appel, R., & Kienzle, W. (2012). Crosstalk cascades for frame-rate pedestrian detection. In European Conference on Computer Vision (ECCV) (pp. 645–659). Springer. https://doi.org/10.1007/978-3-642-33712-3_46

Dollár, P., Appel, R., Belongie, S., & Perona, P. (2014). Fast feature pyramids for object detection. IEEE Transactions on Pattern Analysis and Machine Intelligence, 36(8), 1532–1545. https://doi.org/10.1109/TPAMI.2014.2300479

Gollner, M. J. (2016). Detection and Suppression of Fires: A Cornerstone of Fire Protection Engineering. Fire Technology, 52(5), 1193–1196. https://link.springer.com/article/10.1007/s10694-016-0606-2

Jiji, A. C., Prasad, K. D. V., Jeyaraman, N., Sahu, D. N., Begum, A. Y., & Patil, N. J. (2022). IoT-Based Automatic Forest Fire Detection Based on Machine Learning Approach. Annals of Forest Research, 65(1), 1792–1809. https://www.researchgate.net/publication/365264540_IOT_BASED_AUTOMATIC_FOREST_FIRE_DETECTION_BASED_ON_MACHINE_LEARNING_APPROACH

Kakde, A., Arora, N., & Sharma, D. (2018). Fire detection system using artificial intelligence techniques. International Journal of Research in Engineering, IT and Social Sciences, 8(11), 1–5. https://www.researchgate.net/publication/329642719_Fire_Detection_System_Using_Artificial_Intelligence_Techniques

McNeil, J. G., & Lattimer, B. Y. (2016). Autonomous fire suppression system for use in high and low visibility environments by visual servoing. Fire Technology, 52, 1343–1368.

McNeil, J. G., Starr, J. W., & Lattimer, B. Y. (2013). Autonomous fire suppression system. In Proceedings of the IEEE/ASME International Conference on Advanced Intelligent Mechatronics (AIM) (pp. 1504–1509). IEEE.

Nainar, A., Ishwariya, P., Mythireyeni, L., & Neelaveni, M. (2022). Computer vision–based early fire detection using machine learning. International Research Journal of Engineering and Technology (IRJET), 9(4). https://www.irjet.net/archives/V9/i4/IRJET-V9I4332.pdf

Sekarsari, K., Ikhsan, D., & Marfin. (2019). Design of 2-DOF Arm Robot Control System Using Ultrasonic Sensor. IOP Conf. Series: Materials Science and Engineering, 550(1), 012014. https://doi.org/10.1088/1757-899X/550/1/012014

Suh, Y. (2025). Vision-based detection algorithm for monitoring dynamic change of fire progression. Fire Technology, 61, 2571–2591. https://doi.org/10.1007/s10694-024-01698-3

Wang, H., Fu, X., Yu, Z., & Zeng, Z. (2025). DSS-YOLO: An improved lightweight real-time fire detection model based on YOLOv8. Robots.